NAME: Savan Yeshwanth Rao

CSUID: 2784780

LAB- Ethernet and ARP Report

- The Ethernet address of my computer is: - f8:34:41:5e:34:31

- The destination address of ethernet frame is: -  00:50:e8:04:12:4d.

No, it is not the ethernet address of gaia.cs.umass.edu

It is the address of my Nomadix router, which is the link used to get the subset.

- The hex value of the frame type field is 0x0800. This corresponds to IP protocol.

- The ASCII ‘G’ has 47 bytes from the start of the Ethernet Frame.

- The value of Ethernet source address is: - 00:50:e8:04:12:4d

No, it is not the address of my computer. It is the address of Nomadix router

- The destination address in the Ethernet frame is: - f8:34:41:5e:34:31 is the address of my computer.

- The hex value of the frame type field is 0x0800. This corresponds to IP protocol.

- The ASCII “O” in “OK” has “4f” bytes from start of Ethernet Frame.

- The Internet Address field contains the IP address, the Physical Address field contains the MAC address, and the type field indicates the protocol type.

- The hex decimal value of source is: f8:34:41:5e:34:31

The hex decimal value of destination is: ff:ff:ff:ff:ff:ff

- The hexadecimal value of the two-byte ethernet frame type field is: - 0x0806.

It corresponds to ARP upper layer protocol.

A). ARP opcode field starts 20 bytes from very beginning of the Ethernet frame.

B). The hex value of the opcode field within the ARP payload part of the ethernet frame in which the ARP request was made is 0x0001.

C). Yes, the ARP message contain the IP address of the sender which is – 172.20.5.19

D). The field “Target MAC address” is set to 00:00:00:00:00:00 to question the machine whose corresponding IP address (172.20.1.1) is being queried

- The ARP opcode field begins 20 bytes from the very beginning of the Ethernet frame.

- The hex value of the opcode field within the ARP payload part of the ethernet frame in which the ARP response was made is 0x0002.

- The answer to the earlier ARP request appears in the”Sender MAC address” field, which contains the Ethernet address 00:50:e8:04:12:4d for the sender with IP address 172.20.1.1.

- The hex value for the source address is 00:50:e8:04:12:4d and for the destination is f8:34:41:5e:34:31 .

- Since the ARP request is a broadcast (ff:ff:ff:ff:ff:ff), the ARP reply is directly sent back to the source’s ethernet address. Therefore, there is no reply in this trace, since the machine is not that sent the request but the router.

EXTRA CREDIT

EX-1. The arp command:

arp -s,  InetAddr EtherAddr, allows you to manually add an entry to the ARP cache that resolves the IP address InetAddr to the physical address EtherAddr. What would happen if, when you manually added an entry, you entered the correct IP address, but the wrong Ethernet address for that remote interface?

SOLUTION: ARP protocol is used to get the correct address. Once the router received the destination IP address, even when we have entered an incorrect MAC address, the router will remove IP address from the Ethernet frame and using ARP, would get the correct MAC address of the destination.

EX-2. What is the default amount of time that an entry remains in your ARP cache before being removed? You can determine this empirically (by monitoring the cache contents) or by looking this up in your operation system documentation. Indicate how/where you determined this value.

Solution:  The default amount of time that entry remains in your ARP cache before being removed is 21,600 seconds (6 hours). And we can specify how long an entry needs to remain in the ARP cache using the ARP timeout command.